RNAscope Protocol Using Prefixed Mouse Brain Tissues (Post-48 hour in PFA/NBF)

Prepare Sections

- Set the oven to 60°C.

- Prepare a petri dish with deionized water (DI water), thin brushes for transferring the brain slices, and another light source.

- Using the thin brush, move the brain slice from the plate it arrived in into the petri plate. Gently unfold the brain slice as much as you can. Use two thin brushes if desired.

- Pipette 100 µL of the DI water in the petri plate onto the slide for mounting the brain slices. Treat the slide as three sections: top, middle, and bottom section, large enough to fit 3 brain samples.

- Carefully transfer the brain slice using the thin brush to move it into the water droplet on the slide.

- Continue to unfold the brain slice while gently wiping excess water with a Kimwipe when desired, until the sample is completely flat. Avoid directly touching the sample with Kimwipe.

- Using the thin brush, orient the brain slices in the same manner with image processing purposes in mind.

- Bake the slides at 60°C for 30 in the humidity tray (maximum 20 slides) with the tray lid off and no wet paper towels. After the incubation, adjust the temperature of the oven to 40°C in preparation for the next steps.

- During this incubation time, prepare 1x wash buffer by pouring 60 mL of the 50X RNAscope wash buffer (Ref. No. 320058) with 2.94 L milli-Q water in a 4 L graduated cylinder. Cover the opening with parafilm and invert the cylinder twice over the sink, cupping your hand on the parafilm, to mix.

Fix and Dehydrate Sections

- Place prefix tissues into the slide rack and immerse slides in 10% Neutral Buffer Formalin (10% NBF). Place the container on ice for 15 mins. If using more than 12 slides (humidity tray allows for a maximum of 20 slides), use two slide racks for the rest of the protocol whenever it calls for the use of the slide rack.

- During this incubation time, prepare ethanol (EtOH, Ref. No. BP2818-4) standards. Each slide rack container is to be filled to 100 mL:

| Percentage | EtOH (mL) | DI water (mL) |
| --- | --- | --- |
| 50% | 50 | 50 |
| 70% | 70 | 30 |
| 100% | 100 | 0 |

- Immerse slide in 50% EtOH for 5 mins at room temperature (RT).

- Immerse slide in 70% EtOH for 5 mins at RT.

- Immerse slide in 100% EtOH for 5 mins at RT. Repeat with fresh 100% EtOH.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample. Allow the sample to dry for 5 min at RT. Start the timer when the last slide is wiped.

Pretreat Slides

- Add 5 drops of hydrogen peroxide (Ref. No. 322335) to cover each sample and incubate at RT for 10 mins. For preparations using more than 10 slides, stagger two timers: set one 10 min timer for the first set of ten slides, and a separate 10 min timer for the second set of slides.

- During this incubation time, prepare 1X Target Retrieval Buffer by pouring 70 mL of the 10X Target Retrieval Buffer (Ref. No. 322001) with 700 mL DI water in a 1 L glass beaker and boil on a hot plate.

- Wash slides in DI water by moving the slide rack up and down 5 times.

- Repeat with fresh DI water.

Target Retrieval

- Submerge slides in slide rack in boiling liquid for 5 mins, ensuring the temperature is more than 99°C and less than 105°C with a thermometer.

- Immediately transfer hot slide rack into RT DI water.

- Wash slides in DI water moving the slide rack up and down 5 times.

- Repeat with fresh DI water.

- Wash slides in 100% EtOH, moving the slide rack up and down 5 times.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample. Allow the sample to dry for 5 min at RT. Start the timer when the last slide is wiped.

- During this incubation time, line the humidity tray with paper towels, fill it with DI water, pour out the excess water, and place the tray into the oven at 40°C

- Gently draw 1-2 circles with the hydrophobic barrier pen (Cat. No. H-4000) closely around each sample without touching the sample. Allow the slides to dry for 5 mins at RT Start the timer when the last slide is finished. Avoid pressing too hard to prevent overflowing of the pen ink onto the sample. Depress the pen on the paper towel if the tip is too dry.

- Insert the slides into the humidity tray.

- Quickly add 1 drop of protease III to cover each section.

- Incubate slides in humidity control tray at 40°C for 30 mins.

- During this incubation time, dilute probes, ensuring the probes are cooled to RT before using.

| # Experiments | C1 (uL) | C2 (uL) | C3 (uL) |
| --- | --- | --- | --- |
| 6 | 240 | 5 | 5 |
| 10 | 480 | 10 | 10 |
| 20 | 960 | 20 | 20 |

- Wash slides in DI water moving the slide rack up and down 5 times.

- Repeat with fresh DI water.

Hybridize Probe

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Insert the slides into the humidity tray.

- For the negative control probe (Ref. No. 320871) and the positive control probe (Ref. No. 320881), add 1 drop to cover corresponding samples. For the experimental probe mixture, pipette 30 uL to cover corresponding samples.

- Incubate in a humidity tray for 2 hours at 40°C. After incubation, turn off the oven, discard the paper towels, and rinse the humidity tray with water before storing it.

- Remove slides from oven and submerge in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer. The washing tray requires about 200 mL of wash buffer to completely submerge the samples. Add more if needed.

- Repeat with fresh wash buffer.

Stop

- Transfer the slides to a slide rack and submerge them in 5X saline-sodium citrate (5X SSC, Ref. No. S6639-1L) overnight (ON). Add 25 mL of the 20X to 75 mL of DI water to make 100 mL of 5X SSC to fill the slide rack container.

Hybridize Amp 1

- Line the humidity tray with paper towels, fill it with deionized water (DI water), pour out the excess water, and place the tray into the oven at 40°C.

- Transfer the slides to the humidity tray.

- Wash off 5X SSC by submerging the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of amp 1 (Ref. No. 323101) to cover each sample and incubate in the humidity tray at 40°C for 30 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Hybridize Amp 2

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of amp 2 (Ref. No. 323102) to cover each sample and incubate in the humidity tray at 40°C for 30 mins.

- During this incubation time, prepare more 1X wash buffer by pouring 60 mL of the 50X RNAscope wash buffer (Ref. No. 320058) with 2.94 L milli-Q water in a 4 L graduated cylinder. Cover the opening with parafilm and invert the cylinder twice over the sink, cupping your hand on the parafilm, to mix.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Hybridize Amp 3

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Add 1 drop of amp 3 (Ref. No. 323103) to cover each sample and incubate in the humidity tray at 40°C for 15 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Develop HRP-C1 Signal (Develop only for channels in use/corresponding fluor)

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of HRP-C1 (Ref. No. 323104) to cover each sample.

- Incubate in the humidity tray at 40°C for 15 mins.

- During this incubation time, prepare corresponding fluorophores. Alexa fluor reagents (stock dry pellet) are to be resuspended in 150 µL of DMSO (Ref. No. J66650.AD) before use. Resuspended Alexa fluor reagents are to be diluted at 1:100 in TSA buffer (Ref. No. 322809).

| # Slides | Alexa Fluor (uL) | TSA Buffer (mL) |
| --- | --- | --- |
| 6 | 10 | 0.99 |
| 10 | 20 | 1.98 |
| 20 | 40 | 3.96 |

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Check which dye goes with which probe. Quickly add 30 µL of corresponding diluted Alexa dye, covering each sample.

- Incubate in the humidity tray at 40°C for 30 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of v2 HRP blocker (Ref. No. 323107) to cover each sample.

- Incubate in a humidity tray at 40°C for 15 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Develop HRP-C2 Signal (Develop only for channels in use/corresponding fluor)

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of HRP-C2 (Ref. No. 323105) to cover each sample.

- Incubate in a humidity tray at 40°C for 15 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Check which dye goes with which probe. Quickly add 30 µL of corresponding diluted Alexa dye, covering each sample.

- Incubate in the humidity tray at 40°C for 30 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of v2 HRP blocker (Ref. No. 323107) to cover each sample.

- Incubate in the humidity tray at 40°C for 15 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Develop HRP-C3 Signal (Develop only for channels in use/corresponding fluor)

- Retrieve the prolong glass antifade reagent (Ref. No. P36984) and thaw at RT for 1 hour before use to help prevent bubbles.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of HRP-C3 (Ref. No. 323106) to cover each sample.

- Incubate in a humidity tray at 40°C for 15 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Check which dye goes with which probe. Quickly add 30 µL of corresponding diluted Alexa dye, covering each sample.

- Incubate in the humidity tray at 40°C for 30 mins.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of v2 HRP blocker (Ref. No. 323107) to cover each sample.

- Incubate in the humidity tray at 40°C for 15 mins. After incubation, turn off the oven, discard the paper towels, and rinse the humidity tray with water before storing it.

- Remove slides from oven and immerse the tray in wash buffer for 2 mins at RT, agitating slides slightly by moving the tray left and right in the buffer.

- Repeat with fresh wash buffer.

Prepare and Mount Slides

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe.

- Quickly add 1 drop of DAPI (Ref. No. 323108) to cover each sample.

- Incubate samples for 10-60 (finalized time TBD) mins at RT. For preparations using more than 10 slides, stagger two timers: set one 5 min timer for the first set of ten slides, and a separate 5 min timer for the second set of slides. This is to not be mixed up with the different incubation time used for frozen tissues.

- Wash slides in 1X PBS in a slide rack, moving slides up and down 5 times.

- Repeat with fresh 1X PBS.

- Remove excess liquid by gently flicking slides towards a paper towel and gently wiping around the sample with a Kimwipe. Allow the samples to dry for 5 mins at RT. Start this timer when the last slide is wiped.

- Set the pipette to 60 µL and slowly draw up the prolong glass antifade reagent (Cat. No. P36984). After ensuring there are no bubbles on the tip, slowly add the reagent on each sample, halting before the 1st stop of the pipette to prevent any bubbles from entering the sample. If there are bubbles, use the pipette to remove them.

- Carefully place a #1.5 coverslip (Ref. No. CG15KH1) on each slide while avoiding bubbles:

  - Using forceps with one hand, line the long edge of the coverslip with the long edge of the slide against your index and thumb on the opposite hand.

  - Slowly lower the cover glass and center the cover glass with forceps. Prevent the coverslip from entering the “mounting zone,” indicated by two plus signs at the bottom of the slide (which allows for proper mounting on the microscope).

- Allow the mounting reagent to harden ON in the dark at RT and seal the next day with nail polish.

- Place sample in the 4°C ON until ready for imaging. The samples are most stable for 2 weeks.